DIY Dog Stairs / Bed-Steps Plan Bundle

Planning sheets for safer bed and couch access for older, cautious, or mobility-limited dogs

Includes: stairs-or-ramp decision sheet, measurement worksheet, sample sizing profiles, custom cut-list planner, traction-surface guide, build-day checklist, and first-use fit check.

| Best for: Dogs who can still step up safely but need shorter rises, grippy surfaces, and a stable approach. Use this bundle when: You want a home DIY plan before buying materials or choosing dimensions. | Do not use as a substitute for: Veterinary advice, rehab planning, or commercial engineering review. Switch to a ramp when: The dog drags paws, cannot lift evenly, panics on steps, or needs a gentler angle. |
| --- | --- |

1. Stairs or ramp?

Use stairs when the dog can still make deliberate step-ups with control. Choose a ramp when you need a gentler, more continuous path.

| Check this sign | Leans toward stairs | Leans toward ramp |
| --- | --- | --- |
| Dog can place each paw cleanly | Yes |  |
| Dog hesitates only at the first step | Yes |  |
| Dog has short legs but fair balance | Yes |  |
| Dog slips or scrambles on short rises |  | Yes |
| Dog drags paws or knuckles |  | Yes |
| Dog shows uneven hind-end strength |  | Yes |
| Furniture is high and approach distance is tight | Maybe | Maybe |
| Dog startles easily on narrow surfaces |  | Yes |

Decision rule: if you feel torn, start with the gentler option. A stable ramp is usually easier to re-use across rooms than a steep stair unit.

Quick safety screen

Stop and reassess if the dog refuses the setup after calm training and treats.

Use stairs only if the unit will not tip, slide, or twist under real use.

Plan a top landing that sits flush to the furniture edge or slightly below it - never above it.

2. Measurement worksheet

Measure the actual room, not the product photo. Record the finished floor, not the rug edge.

Room + dog measurements

| A. Finished height to target surface | _____ in |
| --- | --- |
| B. Available floor depth in front of furniture | _____ in |
| C. Clear width available | _____ in |
| D. Dog shoulder width at widest point | _____ in |
| E. Dog body length nose-to-tail base (optional) | _____ in |
| F. Existing slip points nearby | ________________________ |
| G. Preferred side of approach | Left / Center / Right |
| H. Dog currently uses: bed / couch / chair / vehicle | ________________________ |

Sizing targets

| Dimension | Good starting target | Why it matters |
| --- | --- | --- |
| Stair rise | 4.5 - 6 in | Shorter rises are easier for older or cautious dogs. |
| Tread depth | 10 - 14 in | Allows a full paw plant instead of a toe-only step. |
| Usable width | Dog width + 6 - 10 in | Prevents crowding and shoulder bumping. |
| Top landing | Flush or 0 - 1 in below target | Keeps the final step predictable. |

Formula shortcut: number of steps = ceiling(total height / target rise). Then actual rise = total height / number of steps.

3. Sample sizing profiles

Use these as starting profiles, then adjust to your own room and dog.

| Profile | Target height | Steps | Rise each | Tread | Typical fit |
| --- | --- | --- | --- | --- | --- |
| A | 18 in | 3 | 6 in | 12 in | Lower bed, low couch, window bench |
| B | 21 in | 4 | 5.25 in | 12 in | Standard couch or average bed frame |
| C | 24 in | 4 | 6 in | 13 in | Taller bed with enough floor depth |
| D | 27 in | 5 | 5.4 in | 13 in | Tall bed where a shorter rise helps |

Profile notes

When floor depth is limited, do not keep adding steep steps. Move to a ramp or a lower furniture target instead.

Very small dogs often prefer a little more tread depth than expected because they pause and re-balance between steps.

For cautious dogs, the best early win is usually a wider, grippier unit - not a taller one.

4. Custom cut-list planner

This is a homeowner planning sheet. Confirm all dimensions after accounting for your actual material thickness.

| Part | Qty | Size (in) | Notes | Done |
| --- | --- | --- | --- | --- |
| Side panels / stringers | 2 | _____ x _____ | Mirror pair; mark left/right | □ |
| Step treads | _____ | _____ x _____ | Depth should fit full paw plant | □ |
| Risers / front faces | _____ | _____ x _____ | Optional if using open style | □ |
| Top landing panel | 1 | _____ x _____ | Meet furniture edge cleanly | □ |
| Rear brace | 1 | _____ x _____ | Prevents twist or sway | □ |
| Bottom anti-slip feet | 4 | n/a | Rubber pads or grippy base | □ |
| Traction surface strips | _____ | _____ x _____ | Leave no slick gaps | □ |

Starter material list: plywood or furniture board, screws sized for your stock, wood glue if desired, anti-slip feet, traction surface, sandpaper, and edge-softening supplies.

5. Traction-surface guide

The top failure point is a smooth tread. Good traction often matters more than perfect woodworking.

| Surface option | Best use | Watch-outs | Score |
| --- | --- | --- | --- |
| Low-pile carpet runner | Great all-purpose option | Can hold odor if accidents are frequent | High |
| Rubberized stair tread strips | Strong grip on each step | Edges must be bonded flat | High |
| Indoor/outdoor carpet tile | Durable and easy to replace | Check paw feel; some dogs dislike coarse fibers | Medium-high |
| Foam mats | Soft under paw | Can compress and shift if not fixed well | Medium |
| Bare sealed wood | Looks clean | Too slick for this use | Low |

Placement notes

Carry traction all the way to the top landing so the final transfer is not the slickest point.

Round over sharp tread edges and sand splinter-prone corners before adding surface material.

If accidents are likely, favor surfaces that can be replaced in strips instead of redoing the whole unit.

6. Build-day and first-use check

The build is not finished when the screws are in. It is finished when the dog can use the setup calmly, without slipping or surprise.

Build-day checklist

|  | Checklist item |
| --- | --- |
| □ | Confirm finished height, width, and floor depth before the first cut. |
| □ | Dry-fit pieces and test for twist or wobble before final fastening. |
| □ | Sand edges and any paw-contact surfaces. |
| □ | Install anti-slip feet and traction before indoor placement. |
| □ | Push from multiple directions to check for tipping or racking. |

First-use fit check

|  | Checklist item |
| --- | --- |
| □ | Introduce with treats and calm repetition - do not force the dog onto it. |
| □ | Watch whether each step allows a full, stable paw plant. |
| □ | Check for hesitation at the top landing or the final step. |
| □ | If the dog side-steps or widens stance, reassess width and traction. |
| □ | If refusal continues after calm tries, move to a gentler design or a ramp. |

7. Final fit notes

Use this page after a few days of real use. The goal is to improve the setup, not to prove the first draft was correct.

| What the dog did well | __________________________________________ |
| --- | --- |
| Where hesitation showed up | __________________________________________ |
| Any slip or paw-miss points | __________________________________________ |
| Would a wider tread help? | Yes / No / Not sure |
| Would a lower rise help? | Yes / No / Not sure |
| Would a ramp be easier? | Yes / No / Not sure |
| Next adjustment to test | __________________________________________ |

Bonus idea for your storefront: pair this paid bundle with a free caregiver handoff sheet so buyers can keep home setup instructions in one place.

For home DIY planning only. Supervise first use. If the dog shows pain, panic, or worsening mobility, stop and consult a veterinarian.